#include <WiFi.h> #include <HTTPClient.h> #include "DHT.h" // WiFi Credentials char ssid[] = "YOUR_WIFI_NAME"; char password[] = "YOUR_WIFI_PASSWORD"; // Firebase Realtime Database URL String firebaseUrl = "httpsPin Configurations int MQ2_GAS_PIN = 34; int FLAME_PIN = 35; int DHT_PIN = 4; int BUZZER_PIN = 19; int RELAY_PIN = 21; // Initialize DHT Sensor DHT dht(DHT_PIN, DHT11); void setup() { Serial.begin(115200); // Set Pin Modes pinMode(MQ2_GAS_PIN, INPUT); pinMode(FLAME_PIN, INPUT); pinMode(BUZZER_PIN, OUTPUT); pinMode(RELAY_PIN, OUTPUT); // Initialize DHT Sensor dht.begin(); // Connect to WiFi Serial.print("Connecting to WiFi"); WiFi.begin(ssid, password); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } Serial.println("\nConnected!"); } void loop() { // Read Sensor Inputs int gasValue = analogRead(MQ2_GAS_PIN); int flameValue = digitalRead(FLAME_PIN); float tempValue = dht.readTemperature(); float humValue = dht.readHumidity(); // Flame sensor active LOW logic boolean isFire = (flameValue == LOW); // Gas threshold logic boolean isGasHigh = (gasValue > 400); String statusMessage = "SAFE"; // Alert and Relay Control if (isFire || isGasHigh) { statusMessage = "HAZARD"; digitalWrite(BUZZER_PIN, HIGH); digitalWrite(RELAY_PIN, HIGH); } else { digitalWrite(BUZZER_PIN, LOW); digitalWrite(RELAY_PIN, LOW); } // Handle DHT sensor read errors gracefully if (isnan(tempValue) || isnan(humValue)) { Serial.println("Failed to read from DHT sensor!"); tempValue = 0.0; humValue = 0.0; } // Construct JSON Payload String json = "{"; json += "\"gas\":" + String(gasValue) + ","; json += "\"fire\":" + String(isFire ? "true" : "false") + ","; json += "\"temp\":" + String(tempValue) + ","; json += "\"humidity\":" + String(humValue) + ","; json += "\"status\":\"" + statusMessage + "\""; json += "}"; // Send HTTP PUT request to Firebase if (WiFi.status() == WL_CONNECTED) { HTTPClient http; http.begin(firebaseUrl); http.addHeader("Content-Type", "application/json"); int httpCode = http.PUT(json); Serial.print("Data Sent. Response Code: "); Serial.println(httpCode); http.end(); } else { Serial.println("WiFi Disconnected. Retrying..."); } delay(2000); }